Estimation of changes in cyclic lung strain by electrical impedance tomography Proof-of-concept study

Cornejo, R, Iturrieta, P, Olegário, TMM, et al. Estimation of changes in cyclic lung strain by electrical impedance tomography: Proof-of-concept study. Acta Anaesthesiol Scand. 2021; 65: 228– 235. https://doi.org/10.1111/aas.13723

Abstract

Cyclic strain may be a determinant of ventilator-induced lung injury. The standard for strain assessment is the computed tomography (CT), which does not allow continuous monitoring and exposes to radiation. Electrical impedance tomography (EIT) is able to monitor changes in regional lung ventilation. In addition, there is a correlation between mechanical deformation of materials and detectable changes in its electrical impedance, making EIT a potential surrogate for cyclic lung strain measured by CT (StrainCT).

Objectives

To compare the global StrainCT with the change in electrical impedance (ΔZ)

Methods

Acute respiratory distress syndrome patients under mechanical ventilation (VT 6 mL/kg ideal body weight with positive end-expiratory pressure 5 [PEEP 5] and best PEEP according to EIT) underwent whole-lung CT at end-inspiration and end-expiration. Biomechanical analysis was used to construct 3D maps and determine StrainCT at different levels of PEEP. CT and EIT acquisitions were performed simultaneously. Multilevel analysis was employed to determine the causal association between StrainCT and ΔZ. Linear regression models were used to predict the change in lung StrainCT between different PEEP levels based on the change in ΔZ.

Main results

StrainCT was positively and independently associated with ΔZ at global level (P < .01). Furthermore, the change in StrainCT (between PEEP 5 and Best PEEP) was accurately predicted by the change in ΔZ (R2 0.855, P < .001 at global level) with a high agreement between predicted and measured StrainCT.

Conclusions

The change in electrical impedance may provide a noninvasive assessment of global cyclic strain, without radiation at bedside.